# Lab Session: Reading Accelerometer Data and 1D Kalman Filter Implementation

## Objectives

• Read raw accelerometer and gyroscope data from the MPU-6050 sensor using the I²C interface.

• Calibrate the accelerometer to correct sensor bias and compute roll and pitch angles.

• Implement a one-dimensional Kalman filter to fuse accelerometer and gyroscope readings.

• Analyze how the Kalman filter stabilizes the estimated angles compared to raw sensor data.

## Required Hardware

- Teensy 4.1 (or Arduino Uno compatible)

- MPU-6050 IMU module

- Breadboard and jumper wires

- USB cable

- Optional: Oscilloscope or Serial Plotter for data visualization

- Carbon Aeronautics Quadcopter manual

## Part A – Reading Accelerometer and Gyroscope Data

Initialize the MPU-6050 with I²C and configure its accelerometer to ±8g range. Then, read the 6 registers of accelerometer and 6 registers of gyroscope data. Convert the raw values to physical units using the following formulas:

Acc = LSB / 4096

Rate = LSB / 65.5

Compute the roll and pitch angles:

AngleRoll = atan(AccY / sqrt(AccX² + AccZ²)) × (180 / π)

AnglePitch = -atan(AccX / sqrt(AccY² + AccZ²)) × (180 / π)

Perform accelerometer calibration by recording offsets when the sensor is level, and subtract those values from AccX, AccY, and AccZ to minimize bias.

## Part B – One-Dimensional Kalman Filter Implementation

Create a Kalman filter to combine the gyro and accelerometer measurements. The Kalman filter equations are implemented in a function that updates the estimated angle and uncertainty.

void kalman_1d(float KalmanState, float KalmanUncertainty,
               float KalmanInput, float KalmanMeasurement) {
  KalmanState = KalmanState + 0.004 * KalmanInput;
  KalmanUncertainty = KalmanUncertainty + 0.004 * 0.004 * 4 * 4;
  float KalmanGain = KalmanUncertainty / (KalmanUncertainty + 3 * 3);
  KalmanState = KalmanState + KalmanGain * (KalmanMeasurement - KalmanState);
  KalmanUncertainty = (1 - KalmanGain) * KalmanUncertainty;
  Kalman1DOutput[0] = KalmanState;
  Kalman1DOutput[1] = KalmanUncertainty;
}

In the main loop, use the filter to estimate roll and pitch angles from sensor readings, then print both the raw and filtered values to the Serial Monitor for comparison.

## Part C – Analysis and Discussion

• Record 60 seconds of data with the sensor stationary and compare raw vs filtered output.

• Plot both on the Serial Plotter or external tool (e.g., Excel, MATLAB).

• Discuss how the Kalman filter reduces noise and drift from the accelerometer and gyroscope.

## Post-Lab Questions

- Why does integrating gyroscope data cause drift over time?

- How does the Kalman gain determine the influence of the accelerometer versus the gyroscope?

- What happens if the measurement noise variance (3²) is reduced in the code?

- Compare the standard deviation of raw and filtered angle data. Which one is lower?

- Include in your report your plots, your Kalman filter equation, circuit setup photos.